Specifications of VAL

1 - 	Description and physical characteristics

1.1    	The adjustable VAL diffusers shall consist of a circular duct with three (3) rings which guide the airflow with a manual or motorized adjustment mechanism.

1.2    	The adjustment mechanism shall be composed of a deflector, a reduction ring and an adjustable control dial.

1.3    	The circular duct, deflector and reduction ring shall be of galvanised steel with a black finish.

1.4    	Rings and control dial shall be in aluminum with a white (RAL 9003) finish.

2 - 	Performance

2.1 	The performance shall be guarantee by using performance curves or simulation software for critical areas. These shall indicate the loss of pressure and the acoustic power generated and shows a cross-section of the critical airflow trajectory in cooling mode and in heating mode or in isothermal mode with a nominal speed in the occupied zone at the request of the engineer.

3  - 	Connection

3.1 	The diffuser shall be connected directly to the air duct.

4 - 	Balancing

4.1 	The balancing of NAD Klima diffusers shall be executed by a certified ventilation balancing technician with a recognised professional certification.

5 – 	Required quality: NAD Klima VAL model